Instruction for upgrade RT-490 Radio

- Go to menu 54 to check the hardware version

- If it is Ver 1.0 and your radio is GPS version ,  use the file folder: Firmware version 1.03GPSRt-490 GPS Hw V1.0_20220810

- If it is Ver 2.0 and your radio is NoGPS version, use the file folder Firmware version 1.03NoGPS Rt-490_NoGPS_HW V2.0_20220810

- Connect radio and PC with programming cable

- Open the Bootloader pro.exe and Insert the firmware file

- Select correct COM port

- Press P1+OK buttons at the same time power on radio until Green LED shows

- Click upgrade button

NOTE: if radio can’t power on after upgrade, the selected version is mistake, please try with another upgrade file to fix it.