If RT-900/RT-910 was dead after update

Try to fix it as follow:

- CLOSE the firmware updater software.

- Plug the cable into the computer, not into the radio yet.

- Reopen the firmware updater.

- Choose the settings (com port, 9600 baud rate.)

- Select the firmware file.

- TURN OFF the radio.

- Plug the programming cable into the radio.

- Turn the radio on, holding the two side buttons so the screen says UPDATE...

- Click the black text button, not the red text button.

- Wait 1 minutes.

If there is no solved the problem, remove the battery pack and wait for 10 minutes then try the same way again.